| Exp. No.: 6 | Error Correction at Data Link Layer (Hamming Code) |
| --- | --- |

# Aim

To write a program to implement error detection and correction using the Hamming code concept, and to make a test run to input a data stream and verify the error correction feature.

# Required Software / Tools

- Python 3 (or any general-purpose programming language)

# Theory / Concepts

Hamming code is a set of error-correction codes that can be used to detect and correct errors that occur when data is transmitted from a sender to a receiver. It is a technique developed by R. W. Hamming for error correction. Redundant (parity) bits are inserted into the data at specific positions so that, on the receiving end, any single-bit error can be located and corrected by checking the parity bits.

# Procedure

## Sender program:

- Take a text input of any length and convert it to binary.

- Apply the Hamming code concept on the binary data (encoding each 4-bit block into a 7-bit Hamming codeword) to add redundant bits.

- Save this encoded output in a file called "channel.txt".

## Receiver program:

- Read the input from the channel file.

- Apply the Hamming code check on the binary data to check for errors.

- If there is an error, display the position of the error.

- Otherwise, remove the redundant bits, convert the binary data to ASCII, and display the output.

# Program / Code

## Sender Program (sender.py)

# Hamming Code Sender Program

# Converts input text to binary, applies Hamming(7,4) code to each 4-bit # nibble (adding 3 redundant/parity bits), and writes the encoded bit

# stream to a file called "channel.txt".

def text_to_bits(text):

return ''.join(format(ord(ch), '08b') for ch in text)

def hamming_encode(d):

# d = [d1, d2, d3, d4] (4 data bits) d1, d2, d3, d4 = d

p1 = d1 ^ d2 ^ d4 p2 = d1 ^ d3 ^ d4 p4 = d2 ^ d3 ^ d4

# Codeword positions: p1 p2 d1 p4 d2 d3 d4 return [p1, p2, d1, p4, d2, d3, d4]

def encode_stream(bitstring):

# Pad to a multiple of 4 bits while len(bitstring) % 4 != 0:

bitstring += '0'

encoded = ''

for i in range(0, len(bitstring), 4):

nibble = [int(b) for b in bitstring[i:i+4]] codeword = hamming_encode(nibble)

encoded += ''.join(str(b) for b in codeword) return encoded

if  name  == " main ":

text = input("Enter text to send: ") bits = text_to_bits(text) print("Binary form of input :", bits)

encoded_bits = encode_stream(bits) print("Hamming encoded data :", encoded_bits)

with open("channel.txt", "w") as f: f.write(encoded_bits)

print("Encoded data written to channel.txt")

## Receiver Program (receiver.py)

# Hamming Code Receiver Program

# Reads the encoded bit stream from "channel.txt", applies Hamming(7,4) # error checking on each 7-bit codeword, reports the position of any

# single-bit error found, corrects it, then reconstructs and displays # the original ASCII text.

def hamming_decode(codeword):

# codeword = [p1, p2, d1, p4, d2, d3, d4] (positions 1..7) c = codeword[:]

p1, p2, d1, p4, d2, d3, d4 = c

c1 = p1 ^ d1 ^ d2 ^ d4 c2 = p2 ^ d1 ^ d3 ^ d4 c4 = p4 ^ d2 ^ d3 ^ d4

error_pos = c1 * 1 + c2 * 2 + c4 * 4	# 0 = no error if error_pos != 0:

# Flip the erroneous bit (1-indexed position) to correct it c[error_pos - 1] ^= 1

p1, p2, d1, p4, d2, d3, d4 = c data_bits = [d1, d2, d3, d4] return data_bits, error_pos

def bits_to_text(bitstring): chars = []

for i in range(0, len(bitstring), 8): byte = bitstring[i:i+8]

if len(byte) == 8: chars.append(chr(int(byte, 2)))

return ''.join(chars)

if  name  == " main ":

with open("channel.txt", "r") as f: received = f.read().strip()

print("Received data from channel:", received) decoded_bits = ''

for i in range(0, len(received), 7):

codeword = [int(b) for b in received[i:i+7]]

if len(codeword) < 7: continue

data_bits, error_pos = hamming_decode(codeword) if error_pos != 0:

print(f"Error detected in codeword {i // 7 + 1} at bit position

{error_pos} -- corrected.")

decoded_bits += ''.join(str(b) for b in data_bits)

# Remove padding added by the sender, then convert back to text original_bits = decoded_bits[:len(decoded_bits) - (len(decoded_bits) % 8)] text = bits_to_text(original_bits)

print("Recovered original message:", text)

# Input

Sample input text given to the sender program:

Enter text to send: HI

# Output

Run 1 — Channel data received without any error:

Binary form of input : 0100100001001001

Hamming encoded data : 1001100111000010011000011001 Encoded data written to channel.txt

Received data from channel: 1001100111000010011000011001 Recovered original message: HI

Run 2 — A single bit was flipped in channel.txt to simulate a transmission error:

Corrupted channel data: 1011100111000010011000011001

Received data from channel: 1011100111000010011000011001 Error detected in codeword 1 at bit position 3 -- corrected. Recovered original message: HI

# Result

A sender-receiver program pair implementing the Hamming code concept was successfully developed and tested. The receiver correctly recovered the original message when no error was present, and correctly detected and corrected a single-bit error introduced into the channel data, verifying the error correction feature of the Hamming code.